GeneralTouch®

Touch Screen Software For Microsoft DOS

Installation Manual

Revision 1.10

General Touch Technology Co., Ltd

www.generaltouch.com

[D/N: GT-IBL-PMD-2003-001]

PREFACE

Thank you for choosing our RS232, Surface Acoustic Wave touch screen products!

This software is the driver package of GeneralTouch surface acoustic wave touch screen for Microsoft DOS, and this manual contains additional notes about our touch screen software. We hope it will help you to make better use of our touch screen and its software.

If there is any comment, please contact:

Mr. Alex Lee or Mr. YangQin

Address: Sunsing High-Tech Zone, No.1 Shuangxing North Road,

Chengdu, Sichuan, P.R. China

Po : 610091
Tel: +8628-7322694

Fax: +8628-7341133

E-mail: alex@generaltouch.com

yangqin@generaltouch.com

CONTENTS

- Install touch screen software

- Set touch screen configuration

3.  Uninstall touch screen software

4.  Software component

1. Install touch screen software

Do the following to install your touch screen and its software:

1> Turn off your computer.

2> Install your touch screen hardware.

3> Turn on your computer.

4> Insert the installation floppy disk, which is labeled "GeneralTouch SAW touch screen driver version 1.00 for MS-DOS ".

Then execute the following command:

A>GENINST<Enter>

You can look at the installation interface as follow:

Here you can input the installation directory or press “Enter” to select the default one.

The installation program may modify your AUTOEXEC.BAT automatically so that the driver can be loaded automatically when your computer turns on.

Now the GeneralTouch touch screen software is installed in the selected directory.

2. Set touch screen configuration

The setup program will run automatically after the installation finishes, or you can run GENSEUP.EXE at the DOS prompt later.

In this program, you can select serial port, baud rate, size of the touch screen, emulation mouse

mode and mouse button mode, and also calibrate the touch screen.

The configuration data will be stored in the file GENTOUCH.CFG.

The followings are the setup interfaces:

The range of the horizontal size is from 0 to 4096;

The range of the vertical size is from 0 to 4096;

Alternatively, you can run GENTOUCH.COM with command options. The command options are described below:

Usage: GENTOUCH [option]

Option is:

/cx    	communication port no., maybe: x = 1, 2, 3 or 4

/bxxxx 	baud rate of communication, maybe: xxxx = 2400, 4800, 9600 or 19200

/dX,Y  	Set Touch Screen's border to (X, Y)

X is maximum horizontal position (0 – 4096)

Y is maximum vertical position (0 – 4096)

/m+    	Enable mouse emulation function

/m-    	Disable mouse emulation function

/l1    	Click left button on touch

/l2    	Click left button on release

/l3    	Drag

/r     	Remove driver from RAM

/s     	Save setting to configuration file.

When the driver is loaded, these configuration data will be showed on the monitor as follow:

3. Uninstall touch screen software

- Return to the MS-DOS prompt.

- Execute the following command:

C>GENTOUCH /r <Enter> or GENTOUCH /R<Enter>

4. Software component

Our touch screen software is composed of the following programs:

GENINST.EXE	Installation program of GeneralTouch touch screen driver for Microsoft DOS.

INSTALL.INF        Installation information file that the installation program needs, which includes the information of installation.

GENSETUP.EXE	Setup program of GeneralTouch touch screen, it will setup the configuration of the driver, such as serial port, baud rate and emulation mode, etc.

It also includes the calibration function.

GENTOUCH.COM    The driver of GeneralTouch touch screen, which provides the interface between 	the touch screen and application programs

GENTOUCH.CFG		The configuration file of the touch screen driver.

GTTMANUAL.doc	It's me!

Copyright 2002-2004 General Touch Technology Co., Ltd.

All rights reserved.

March, 2002 By Alex Lee